Validation of the Segway Model and Lyapunov-Based Stabilizing Controller

Verification of AIDA_project.ipynb and adaptation of Aguilar-Ibañez et al. (2005)

Summary

This report does two things. First, it re-derives the segway dynamics from the Lagrangian and checks each step of the derivation in AIDA_project.ipynb — the conclusion is that the symbolic dynamics in the notebook are correct, with a small markdown typo and two minor code issues listed in §7. Second, it adapts the Lyapunov-based controller of Aguilar-Ibañez, Gutiérrez Frias and Suárez Castañón (2005) from the cart-pole to the segway. The result is a smooth state-feedback law u(θ, θ̇, ψ, ψ̇) that locally asymptotically stabilizes the unstable upright equilibrium, with the domain of attraction characterized by an explicit sublevel set of the constructed Lyapunov function.

1.  Model and verification of the notebook derivation

1.1  Geometry and kinematics

Let θ denote the body tilt angle measured from the upward vertical, and φ the rotation of the wheel about its centre (positive in the direction that rolls the segway in the +x direction). The wheel has radius r and rolls without slipping on flat ground, so the wheel-centre coordinate is xᴡ = rφ at constant height yᴡ = r. The body's centre of mass is at distance l from the wheel axle, giving

xᴬ = rφ + l sinθ,     yᴬ = r + l cosθ.     (1)

Differentiating once and squaring, the body's translational speed satisfies

vᴬ² = r² φ̇² + l² θ̇² + 2 r l cosθ · φ̇ θ̇.     (2)

This matches the symbolic result in cell 7 of the notebook. The preceding markdown cell (cell 6) has a small typo: it omits the factor of 2 on the cross term. The sympy expression in cell 7 is correct; only the displayed LaTeX in cell 6 needs to read 2 r l cosθ · φ̇ θ̇.

1.2  Kinetic and potential energy

With mᴬ, Iᴬ the body mass and centroidal inertia, and mᴡ, Iᴡ the wheel mass and inertia about its centre, the total kinetic energy is

T = ½ (Iᴬ + mᴬ l²) θ̇² + ½ (Iᴡ + (mᴡ + mᴬ) r²) φ̇² + mᴬ r l cosθ · φ̇ θ̇.     (3)

The potential energy of the body is V = mᴬ g (r + l cosθ); the additive constant mᴬ g r does not affect the dynamics, and the wheel centre is at constant height so it contributes nothing.

Introduce the lumped parameters

α = Iᴬ + mᴬ l²,     β = mᴬ r l,     γ = Iᴡ + (mᴡ + mᴬ) r²,     D = mᴬ g l.     (4)

1.3  Equations of motion

Applying the Euler–Lagrange equations to L = T − V, the configuration q = (θ, φ)ᵀ satisfies

M(θ) q̈ + C(θ, θ̇) q̇ + G(θ) = τ,     τ = (u, −u)ᵀ,     (5)

with

M(θ) = [ α,  β cosθ ; β cosθ,  γ ],     (6)

C(θ, θ̇) q̇ = (0, −β sinθ · θ̇²)ᵀ,     G(θ) = (−D sinθ, 0)ᵀ.     (7)

Componentwise, the two scalar equations are

α θ̈ + β cosθ φ̈ − D sinθ = u,     (8a)

β cosθ θ̈ + γ φ̈ − β sinθ · θ̇² = −u.     (8b)

A symbolic re-derivation independent of the notebook reproduces (8) exactly; in particular the Coriolis cross-term in the θ-equation cancels against the corresponding partial of L, leaving β sinθ · θ̇² only in the wheel equation. This confirms the symbolic output of cells 7–32 of AIDA_project.ipynb.

One technical point: the notebook obtains C via C = eq.jacobian(dq)/2. This formula is not the standard Christoffel decomposition (which symmetrizes the partials of M with respect to q), but for this system it produces a pair (C, G) such that Cq̇ + G is correct. So the equations of motion are right, but C is non-standard and the usual skew-symmetry property of Ṁ − 2C does not hold. For the Lyapunov design in §4 this is immaterial; only M itself enters.

1.4  Upright equilibrium

Setting θ = 0 and θ̇ = φ̇ = 0 in (8) gives u = 0, regardless of the wheel angle φ. Hence the upright posture is an equilibrium for every value of φ — φ is a cyclic coordinate. This is exactly what the notebook checks in cell 47 with s_des = [0, φ, 0, 0], and the result is consistent. The later choice s_des = [0, π, 0, 0] simply linearizes at the same upright posture with an arbitrary wheel offset π; nothing in the linearization changes.

2.  A coordinate change that exposes the cart-pole structure

The cart-pole controller of Aguilar-Ibañez et al. (eqs. 1–6) is derived for an input that acts only on the cart coordinate. In the segway, the motor torque appears as τ = (u, −u)ᵀ — it acts on both coordinates. The cleanest way to re-use the paper's design is to first change variables so that the new input acts on a single coordinate.

Define the relative wheel–body angle

ψ := φ − θ,     q̃ := (θ, ψ)ᵀ = T q,     T = [1, 0 ; −1, 1].     (9)

Because T is constant, the new kinetic energy is ½ q̃̇ᵀM̃(θ) q̃̇ with M̃ = T⁻ᵀ M T⁻¹, and the new generalized force is τ̃ = T⁻ᵀ τ = (0, −u)ᵀ — the input now acts only on the actuated coordinate ψ.

A direct calculation gives

M̃(θ) = [ a(θ),  b(θ) ; b(θ),  c ],     (10)

a(θ) = α + 2β cosθ + γ,     b(θ) = γ + β cosθ,     c = γ,     (11)

and the Euler–Lagrange equations in the new coordinates become

a(θ) θ̈ + b(θ) ψ̈ − β sinθ · θ̇² − D sinθ = 0,     (unactuated)     (12a)

b(θ) θ̈ + c ψ̈ − β sinθ · θ̇² = −u.     (actuated)     (12b)

The structure now mirrors equation (3) of the paper. The differences are: (i) the off-diagonal of M̃ is γ + β cosθ rather than cosθ; (ii) the leading coefficient of θ̈ depends on θ — which is the source of the additional β sinθ · θ̇² term in (12a) (absent in the cart-pole because there a is constant). The determinant of M̃ is

Δ(θ) = a c − b² = α γ − β² cos²θ > 0,     (13)

for all θ, since β² = mᴬ² r² l² < α γ by the Cauchy–Schwarz/AM-GM inequality applied to the body and wheel rotation–translation couplings.

3.  Partial feedback linearization

Following Spong's collocated PFL idea (which is also what the paper does in eq. 4), introduce a virtual input v through

u = β sinθ · θ̇² − [ b(θ) sinθ (D + β θ̇²) + Δ(θ) v ] / a(θ),     (14)

which is well-defined because a(θ) > 0. Substituting (14) in (12) gives

θ̈ = [ sinθ (D + β θ̇²) − b(θ) v ] / a(θ),     (15a)

ψ̈ = v.     (15b)

This is the segway analogue of equation (5) of the paper. Two structural differences from the cart-pole remain visible: the coefficient of v in (15a) is b/a, not cosθ; and the drift has an additional Coriolis-like term β sinθ · θ̇²/a. Both differences propagate through the design but do not prevent it.

4.  A Lyapunov function for the upright equilibrium

4.1  Choosing the auxiliary variable ξ

Imitating the paper's ξₓ = q + kᴘ sinθ, we look for a function of the actuated coordinate plus a function of θ alone, so that the result has a clean derivative along the closed-loop flow. The natural choice for this system is

ξ := ψ + kᴘ (γ θ + β sinθ)   ⇒   ξ̇ = ψ̇ + kᴘ b(θ) θ̇.     (16)

In the cart-pole the analogous integrand is cosθ with primitive sinθ; here the integrand is b(θ) = γ + β cosθ with primitive γ θ + β sinθ. The extra γ θ piece is linear in θ (and hence unbounded), but the controller only operates in a bounded region around the upright posture, so this is not an issue.

4.2  A drift-conserved energy Φ

Following the paper's recipe, we seek a function Φ with two properties: (a) its time derivative vanishes for the drift v = 0, and (b) it produces exactly v ξ̇ when v ≠ 0. The ansatz

Φ(θ, θ̇, ψ̇) = kᴘ D (1 − cosθ) − ½ kᴘ a(θ) θ̇² + ½ ψ̇²     (17)

does the job. A direct computation along (15) gives

Φ̇ = v · ( ψ̇ + kᴘ b(θ) θ̇ ) = v · ξ̇.     (18)

Two ingredients are responsible for (18): the coefficient kᴘ a(θ) in front of ½ θ̇² exactly compensates the β sinθ · θ̇² Coriolis term in (15a); and the coefficient kᴘ D in front of (1 − cosθ) matches the D sinθ gravity contribution. This is the segway analogue of equations (9)–(10) of the paper.

4.3  Lyapunov function and its derivative

Take the candidate

V = ½ kɪ ξ² + ½ ξ̇² + kʟ Φ,     (19)

with positive design gains kɪ, kᴘ, kʟ. Differentiating along (15) and using (18),

V̇ = ξ̇ · [ kɪ ξ + ξ̈ + kʟ v ] = ξ̇ · [ η(x) + δ(θ) v ],     (20)

where, after expanding ξ̈ using (15),

η(x) = kɪ ξ + kᴘ sinθ [ D b(θ) − β θ̇² (α + β cosθ) ] / a(θ),     (21)

δ(θ) = kʟ + 1 − kᴘ b(θ)² / a(θ).     (22)

So long as δ(θ) ≠ 0 in the operating region, the feedback

v = − ( ξ̇ + η(x) ) / δ(θ)     (23)

yields

V̇ = − ξ̇² ≤ 0.     (24)

The motor torque is recovered from (14) and (23):

u = β sinθ · θ̇² − [ b(θ) sinθ (D + β θ̇²) + Δ(θ) v ] / a(θ).     (25)

5.  Stability and domain of attraction

5.1  Where V is positive definite

V is the sum of three nonnegative-looking pieces and one indefinite piece (the −½ kᴘ a θ̇² inside Φ). Expanding V in (θ, ψ, θ̇, ψ̇) and collecting quadratic terms, the position quadratic form has determinant kɪ kʟ kᴘ D > 0 — always positive definite. The velocity quadratic form, however, has determinant

¼ kᴘ kʟ [ kᴘ b(θ)² − (1 + kʟ) a(θ) ] = − ¼ kᴘ kʟ a(θ) δ(θ).     (26)

So V is positive definite at the upright posture iff δ(0) < 0, i.e.

kᴘ (γ + β)² > (1 + kʟ) (α + 2β + γ).     (27)

Equation (27) is the gain-tuning condition. (For the cart-pole, with α→1, β→cosθ, γ→1+δ, it reduces to kᴘ > 1 + kʟ, which is the paper's condition.) Define

θˢ := smallest |θ| > 0 such that δ(θ) = 0,     (28)

and the open set

Ω := { (θ, ψ, θ̇, ψ̇) : |θ| < θˢ,  V(x) < Kˢ }.     (29)

On the boundary surface |θ| = θˢ, the velocity quadratic form of V is positive semi-definite (its determinant (26) vanishes), so its minimum over ψ, θ̇, ψ̇ is attained at θ̇ = ψ̇ = 0 and ψ = − kᴘ (γ θˢ + β sinθˢ), giving

Kˢ = kʟ kᴘ D (1 − cosθˢ).     (30)

By construction, any initial state x₀ ∈ Ω stays in Ω forever: (24) shows V is non-increasing along trajectories, and Kˢ is the lowest value of V on the boundary surface. In particular δ(θ(t)) ≠ 0 for all t ≥ 0, so the controller (23)–(25) remains well-defined.

5.2  LaSalle's invariance theorem

Let S = { x ∈ Ω : ξ̇(x) = 0 } be the set where V̇ = 0. By LaSalle, every trajectory starting in Ω converges to the largest invariant subset M ⊆ S. We claim M = {0}.

On a trajectory in M, ξ is constant; call it ξ̄. Differentiating ξ̇ = 0 once more gives ξ̈ = 0, hence from (20)

η(x) + δ(θ) v = − ξ̇ = 0   on M.     (31)

Substituting η from (21) and the explicit form of ξ̈ derived from (15) (which is ξ̈ = (a − kᴘ b²) v / a + drift), the resulting linear equation in v has the θ-independent solution

v = v̄ := − kɪ ξ̄ / kʟ,   constant on M.     (32)

Since ψ̈ = v̄ is constant, ψ̇(t) = ψ̇(0) + v̄ t. But Ω is bounded, so ψ̇ is bounded on the trajectory, which forces v̄ = 0, hence ξ̄ = 0 (because kɪ > 0), and v ≡ 0 on M.

With v = 0, (21) gives η = 0, which reduces to

kᴘ sinθ [ D b(θ) − β θ̇² (α + β cosθ) ] / a(θ) = 0.     (33)

Either sinθ = 0 (so θ = 0 inside |θ| < θˢ ≤ π/2), or

θ̇² = D b(θ) / [ β (α + β cosθ) ]  := f(θ).     (34)

Suppose (34) holds on the trajectory. Differentiating it once and substituting θ̈ from the drift dynamics ((15a) with v = 0) yields the algebraic identity γ − α = 2 (α + β cosθ), which depends on θ and cannot hold along a non-trivial arc. So sinθ = 0, i.e. θ = 0.

With θ = 0, θ̇ = 0, ξ̄ = 0 implies ψ = 0 and ξ = 0; together with ξ̇ = 0, then ψ̇ = 0. So M = {0}, and by LaSalle's invariance principle every trajectory in Ω converges to the upright equilibrium.

5.3  Proposition (segway analogue of Proposition 1 of the paper)

Consider the segway model (5)–(8) controlled by the law (25) (equivalently (14) together with (23)), with positive gains kᴘ, kɪ, kʟ satisfying (27). Let θˢ and Kˢ be defined by (28) and (30). Then the closed-loop system is locally asymptotically stable at the origin, and the open set Ω of (29) is contained in the domain of attraction.

6.  The complete control law

Gathering everything above, the controller can be implemented as the following step-by-step computation, given measurements θ, φ, θ̇, φ̇:

- Set the relative angle ψ = φ − θ  and  ψ̇ = φ̇ − θ̇.

- Compute the configuration-dependent scalars a, b, Δ, δ from (11), (13), (22).

- Form ξ = ψ + kᴘ (γ θ + β sinθ) and ξ̇ = ψ̇ + kᴘ b θ̇.

- Form the function η of (21) and the virtual input v = −(ξ̇ + η) / δ.

- Apply the motor torque u from (25); equivalently τ = (u, −u)ᵀ on body and wheel respectively.

In one closed-form expression (substituting (23) into (14)):

u = β sinθ · θ̇² + [ Δ(θ) (ξ̇ + η) / δ(θ) − b(θ) sinθ (D + β θ̇²) ] / a(θ).     (35)